# Equipment Startup Standard Operating Procedure

Document ID: SOP-EQUIP-001

Version: 1.0 | Effective Date: 2026-01-15 | Author: Operations Engineering

# 1. Purpose

This Standard Operating Procedure defines the safe and consistent startup sequence for the Automated Packaging Line (Model XR-500). It ensures that all operators follow a verified process to bring the equipment from a cold or standby state into full production operation while maintaining safety and quality standards.

# 2. Scope

This procedure applies to all authorized operators, maintenance technicians, and shift supervisors who are responsible for starting the Automated Packaging Line (XR-500) in Building C, Production Area 3. It covers cold start, warm start, and restart-after-fault scenarios.

This procedure does not apply to the auxiliary conveyor systems or the palletizing robot, which are covered under separate SOPs.

# 3. Safety Precautions

Before attempting any startup procedure, all personnel must review and comply with the following safety requirements.

| Category | Requirement | Verification |
| --- | --- | --- |
| PPE | Hard hat, safety glasses, steel-toe boots, cut-resistant gloves | Pre-shift safety check |
| Lockout/Tagout | Verify all LOTO devices removed by authorized personnel only | LOTO log sign-off |
| Emergency Stop | Confirm E-Stop buttons are functional before startup | Physical test at each station |
| Fire Suppression | Verify fire suppression system is armed and indicator is green | Panel indicator check |
| Area Clearance | Confirm no personnel inside guard zones | Walk-around inspection |

# 4. Pre-operation Checklist

Complete all items below before proceeding to the Startup Procedure. Record the result of each check in the Maintenance Log System (MLS).

| Item No. | Check Item | Acceptance Criteria |
| --- | --- | --- |
| 4.1 | Utility power supply voltage | 380V +/- 5% (measured at main panel) |
| 4.2 | Compressed air supply pressure | 6.0 - 7.5 bar at inlet manifold |
| 4.3 | Hydraulic fluid level | Between MIN and MAX marks on reservoir gauge |
| 4.4 | Lubrication points greased | All 12 grease points per maintenance schedule |
| 4.5 | Sensor alignment check | All photoelectric sensors showing green status |
| 4.6 | Safety guard doors closed and latched | All 4 guard door interlocks showing closed |
| 4.7 | HMI software version verified | Version 3.2.1 displayed on startup screen |
| 4.8 | Previous shift maintenance notes reviewed | No open critical work orders |

# 5. Startup Procedure

## 5.1 Cold Start Sequence

Use this sequence when the equipment has been powered off for more than 4 hours or after a complete maintenance shutdown.

Step 1: Turn the main disconnect switch to ON. Wait 30 seconds for the PLC to complete self-diagnostics.

Step 2: On the HMI home screen, press "System Initialize". The status bar will show "Initializing..." for approximately 45 seconds.

Step 3: Verify the HMI displays all subsystems as READY (green). If any subsystem shows FAULT (red), refer to Section 6.

Step 4: Press "Heat Sealer Preheat" and set temperature to 180 degrees C. Wait until the temperature indicator turns green (approximately 3 minutes).

Step 5: Load the product recipe from the recipe library. Confirm the recipe name and version number match the production order.

Step 6: Run the dry-cycle test: press "Dry Cycle" and observe 5 complete cycles. Verify no abnormal sounds, vibration, or misalignment.

Step 7: Load packaging material onto the unwind stand. Thread the film through the guide rollers and sealing jaws per the threading diagram posted at Station 2.

Step 8: Press "Production Ready" to enable the line for product feed.

## 5.2 Warm Start Sequence

Use this sequence when the equipment has been in standby mode for less than 4 hours and no maintenance has been performed.

Step 1: On the HMI, press "Resume from Standby".

Step 2: Verify all subsystem status indicators are green.

Step 3: Confirm the heat sealer temperature is within the acceptable range (175-185 degrees C).

Step 4: Run a 2-cycle dry test to verify mechanical alignment.

Step 5: Press "Production Ready" to enable the line.

## 5.3 Restart After Fault

Use this sequence when the equipment has stopped due to a fault condition. Always investigate and resolve the root cause before restarting.

Step 1: Review the fault log on the HMI (Menu > Fault History > Last 10 Events).

Step 2: If the fault requires physical intervention, apply LOTO before entering the guard zone.

Step 3: After resolving the fault, clear the fault code on the HMI: press "Acknowledge Fault" then "Clear Fault".

Step 4: Press "System Reset" and verify all subsystems return to READY status.

Step 5: Run a 3-cycle dry test to confirm the fault condition is resolved.

Step 6: Resume production by pressing "Production Ready".

# 6. Abnormal Situation Handling

If any abnormal condition is observed during startup or operation, follow the troubleshooting guidance below. If the condition is not listed or cannot be resolved within 15 minutes, contact the on-call maintenance technician.

| Situation | Possible Cause | Immediate Action | Escalation |
| --- | --- | --- | --- |
| HMI shows COMM FAULT on one subsystem | Loose network cable or module failure | Check Ethernet connections at the affected module. Power-cycle the module. | If persists after 2 attempts, call Maintenance. |
| Heat sealer temperature will not reach setpoint | Heating element degradation or thermocouple fault | Verify thermocouple connection. If temperature is below 150 degrees C after 5 minutes, stop and report. | Replace heating element per PM schedule. |
| Film tracking alarm during threading | Film roll misaligned or edge sensor dirty | Re-center the film roll on the unwind stand. Clean the edge sensor with lint-free cloth. | If sensor replacement needed, submit work order. |
| Dry cycle test shows product misalignment | Guide rail out of position or product infeed timing off | Adjust guide rails per the alignment jig. Recalibrate infeed timing on HMI. | If mechanical wear suspected, escalate to Maintenance. |
| Emergency Stop activated unexpectedly | Stuck E-Stop button or wiring fault | Inspect all E-Stop buttons for physical damage. Reset by twisting clockwise. | If wiring fault, apply LOTO and call Electrical Maintenance. |
| Subsystem shows FAULT but no fault code | PLC communication timeout | Press "System Reset". If fault persists, power-cycle the PLC module. | If unresolved, contact Controls Engineering. |

# 7. Shutdown Procedure

## 7.1 Normal Shutdown

Use this sequence at the end of a production shift or when the line will be idle for more than 4 hours.

Step 1: Stop product feed to the infeed conveyor. Wait for all products to clear the sealing section.

Step 2: Press "End Production" on the HMI. The system will complete the current cycle and stop.

Step 3: Press "Heat Sealer Cooldown" and wait until the temperature drops below 60 degrees C (approximately 8 minutes).

Step 4: Remove any remaining packaging material from the unwind stand and sealing jaws.

Step 5: Press "System Shutdown" on the HMI. Wait for the confirmation message.

Step 6: Turn the main disconnect switch to OFF.

Step 7: Complete the shift handover log in the Maintenance Log System.

## 7.2 Emergency Shutdown

Use this sequence ONLY when there is an immediate risk to personnel safety or equipment integrity.

Step 1: Press the nearest Emergency Stop button immediately.

Step 2: If there is a fire, activate the fire alarm and evacuate per the Building C Emergency Plan.

Step 3: Do NOT attempt to restart the equipment until the area has been inspected and cleared by the Shift Supervisor.

# 8. Maintenance Records

All startup, shutdown, and fault events must be recorded in the Maintenance Log System (MLS). The following table summarizes the required record-keeping activities.

| Event Type | Record Required | Retention Period |
| --- | --- | --- |
| Cold Start | Operator name, date/time, checklist results, any deviations | 2 years |
| Warm Start | Operator name, date/time, subsystem status summary | 1 year |
| Fault Event | Fault code, root cause, corrective action, operator name | 5 years |
| Emergency Shutdown | Reason, operator name, date/time, supervisor sign-off | 5 years |
| Scheduled Maintenance | Technician name, work performed, parts replaced, next due date | Life of equipment |

--- End of Document ---